# Vespa Clock Specification

## General

The module has a 160x68 LCD display, a RTC module, the microprocessor board is an ESP32 C3 SuperMini.

There are two pushbuttons, A (left) and B (right) mounted which can be pressed by the rider using the pre-existing pushbutton rubbers that the traditional built in clock had.

The unit is powered by 12v supply from the bike when ignition is switched on. When the ignition is off, the unit is off and the screen blank.

## Power On

On power up (ignition on) the board will follow this sequence (Setup):

- Initialise

- Display Vespa Logo for 4 seconds

- Check for valid settings being saved – apply default settings if not

- Enter Clock mode

## Normal Operation

After initialisation the clock enters Clock Mode.

The A button is used to switch mode. Once it switches mode, it must be released and pressed again to switch mode again. Holding it down will not rapidly go through modes.

Clock Mode

The LCD shows the current time in the chosen format (12 or 24hr) constantly.

The time is read immediately from the RTC on mode entry and then every 1 second thereafter and the display updated if it has changed.

The colon is shown in this mode.

If button A is pressed in this mode, the device enters Date Mode.

Button B has no effect.

Temperature Mode

The current temperature is show in Celsius or Fahrenheit according to preferences.

The temperature is shown right justified. The rightmost digit is always used to display the degrees symbol (top 4 segments of digit on to show a square). If the temperature is negative the middle segment of the leading digit is used to show a minus symbol.

The colon is not shown in this mode.

This shows for 6 seconds and then returns to Clock Mode.

If button A is pressed in this mode, the device enters Set Mode - Hours.

Button B has no effect.

Set Mode – All

If no button is pressed when in any set mode for 10 seconds, the device reverts to Clock mode.

Whenever exiting any set mode, by timeout or button press, the selected setting is saved and if the time has been altered, the RTC is updated.

Set Mode - Hours

The current hour is shown in either 24 or 12 hour format according to the preferences. This shows in the leftmost 2 digits as would appear in clock mode.

If in 12 hour mode, the third digit shows A or P for AM or PM respectively.

The colon is shown in this mode.

Pressing the B button advances the hour by 1, holding B down will advance the hour once every half-second. If the hour advances past 23 (24h mode) or 11PM (12 hour mode) it rolls around to 00 or 12AM respectively.

If button A is pressed in this mode, the device enters Set Mode - Minutes.

Set Mode - Minutes

The current minute is shown in the rightmost 2 digits as would appear in clock mode, always two digits (00 – 59).

The colon is shown in this mode.

Pressing the B button advances the minute by 1, holding B down will advance the minute once every half-second. If the hour advances past 59 it rolls back to 00.

If button A is pressed in this mode, the device enters Set Mode - Hour Format.

Set Mode – Hour Format

If in 12 hour mode, the leftmost two digits show 12, else they show 24.

The third digit shows “h”.

The colon is not shown in this mode.

Pressing the B button toggles between 12 and 24.

If button A is pressed in this mode, the device enters Set Mode - Temperature.

Set Mode – Temperature

The leftmost two digits show “tE”

If in Celsuis mode, the third digit shows C, else it shows F.

The colon is not shown in this mode.

Pressing the B button toggles between C and F.

If button A is pressed in this mode, the device enters Clock Mode.

## Preferences

Settings are:

- 12/24 hour display format (default 12)

- Temp C/F (default C)